LSPU Self-Paced Learning Module (SLM)

| Course | ITEP 309 Networking 2 |
| --- | --- |
| Sem/AY | First Semester/2020-2021 |
| Module No. | 3 |
| Lesson Title | Routing Information Protocol |
| Week Duration | 4-5 |
| Date | Oct 26 to November 06, 2020 |
| Description of the Lesson | This lesson will discuss on Routing Information Protocol that focuses on tracing network hops in tracing where the network traffic proceeds from source to its destination. |

Learning Outcomes

| Intended Learning Outcomes | Students should be able to meet the following intended learning outcomes: - Learn and understand the routing information protocol - To demonstrate the configuration of routing information protocol - To identify the usage and importance of routing information protocol on a corporate network - To simulate different scenarios in implementing routing information protocol |
| --- | --- |
| Targets/ Objectives | At the end of the lesson, students should be able to: - Explain the routing information protocol - Explain the configuration of routing information protocol - Understand the usage and importance of routing information protocol on a corporate network - To understand the simulation of different scenarios in implementing routing information protocol |

Student Learning Strategies

| Online Activities (Synchronous/ Asynchronous) | - Lecture presentation uploaded in Google Classroom Students will be instructed to download lecture presentations with narrations / pre-recorded lecture presentation uploaded in Google Classroom. For further instructions, refer to your Google Classroom and see the schedule of activities for this module. - Learning Guide Questions: - What is Routing Information Protocol? - What is Network Self-Discovery? - What is the commands used in tracing the hops of a specific network traffic? Note: The insight that you will post on online discussion forum using Learning Management System (LMS) will receive additional scores in class participation. |
| --- | --- |
| Offline Activities (e-Learning/Self-Paced) | Lecture Guide Routing Information Protocol (RIP) ▪ Interior Gateway Protocol ▪ Distance-vector protocol using hop count ▪ Maximum hops of 15, 16 is infinite ▪ Oldest dynamic routing protocol, provides updates every 30 seconds ▪ Easy to configure and runs over UDP RIP Characteristics - The Routing Information Protocol (RIP) is a Distance Vector routing protocol - It uses hop count as its metric - The maximum hop count is 15 - It will perform Equal Cost Multi Path, for up to 4 paths by default RIPv2 vs RIPv1 - RIPv1 is a legacy protocol which is not typically used anymore (although it is still supported on Cisco routers) - RIPv1 does not send subnet mask information with routing updates so Variable - Length Subnet Masking (VLSM) is not supported. RIPv2 does support VLSM - RIPv1 updates are sent every 30 seconds as broadcast traffic - RIPv2 uses multicast address 224.0.0.9 RIPv2 supports authentication, RIPv1 does not RIPng RIPng (RIP next generation) supports IPv6 networks RIPv2 Configuration The ‘network’ command should reference a classful network. No subnet mask is specified. Auto-Summary - RIP will automatically summarise routes to the classful boundary by default - For example, 192.168.10.1/30 will be advertised as 192.168.10.0/24 - 172.16.10.1/30 will be advertised as 172.16.0.0/16 - This is almost never desirable Manual Summarization Manual summarisation gives you control of exactly how you summarise The individual summarised routes are not advertised - only their summary route RIPv2 Verification – show ip protocols RIPv2 Verification – show run ¦ section rip RIPv2 Verification – show ip route RIPv2 Verification – show ip rip database Passive Interfaces - Passive interfaces work differently in RIP than other routing protocols - With other routing protocols, a passive interface will not send out or listen for routing updates - The network configured on the interface will be advertised to other peer routers running the routing protocol - In RIP, a passive interface does not send out updates but it does listen to incoming updates from other RIP speaking neighbors - The router can receive updates on the passive interface and use them in the routing table. Default Route Injection Default Route Injection Verification RIP Default Timers - Update: The router sends updates every 30 seconds. - Invalid: After no updates for 180 seconds the route becomes invalid. - Hold Down: The hold down timer is used to stabilize the network, it starts when the invalid timer completes. When a route enters hold down, it can't be installed even if there is a new route with a better metric. 180 seconds by default. - Flush: 240 seconds from the last update the route is flushed. - The timers can be changed to achieve faster convergence times - Be careful with this as it can introduce instability if the timers are set too low - All routers in the network should have the same timer settings - The update timer must be lower than the other timers. Sample Laboratory Infrastructure |
|  | Engaging Activities - How do you define Routing Information Protocol? __________________________________________________________________________________ __________________________________________________________________________________ __________________________________________________________________________________ __________________________________________________________________________________ __________________________________________________________________________________ __________________________________________________________________________________ __________________________________________________________________________________ - How do you explain network hops? __________________________________________________________________________________ __________________________________________________________________________________ __________________________________________________________________________________ __________________________________________________________________________________ __________________________________________________________________________________ __________________________________________________________________________________ __________________________________________________________________________________ - How do you check for directly connected interfaces? __________________________________________________________________________________ __________________________________________________________________________________ __________________________________________________________________________________ __________________________________________________________________________________ __________________________________________________________________________________ __________________________________________________________________________________ __________________________________________________________________________________ __________________________________________________________________________________ __________________________________________________________________________________ __________________________________________________________________________________ __________________________________________________________________________________ - How do you trigger the Routing Information Protocol auto summary? __________________________________________________________________________________ __________________________________________________________________________________ __________________________________________________________________________________ __________________________________________________________________________________ __________________________________________________________________________________ __________________________________________________________________________________ __________________________________________________________________________________ __________________________________________________________________________________ __________________________________________________________________________________ - How do you check how many hops it will take for the source to reach the destination? __________________________________________________________________________________ __________________________________________________________________________________ __________________________________________________________________________________ __________________________________________________________________________________ __________________________________________________________________________________ __________________________________________________________________________________ __________________________________________________________________________________ __________________________________________________________________________________ __________________________________________________________________________________ |

Performance Tasks

Performance Task 2 Direction: Provide a one-liner explanation for the terminologies listed below. Terminologies / sources; Idea or personal explanation. Tracert. Rip database. RIPng. Auto-summary. Network <network-address>.

Understanding Directed Assess

Learning Resources

- https://www.flackbox.com/cisco-ccna-lab-options - https://www.flackbox.com/#elementor-action%3Aaction%3Dpopup%3Aopen%26settings%3DeyJpZCI6IjEwMzk4IiwidG9nZ2xlIjpmYWxzZX0%3D - https://www.youtube.com/channel/UCK4ji45I-zxeWXAFKmu3p6Q - https://study-ccna.com/ - https://www.cybrary.it/course/cisco-ccna/ - https://www.udemy.com/course/ccna-complete/ - https://www.udemy.com/course/networkplus/